Micro Firmware, Inc.

Phoenix Technologies BIOS Support & Upgrades

405-321-8333  - FAX: 405-321-8342

------------------------

Phoenix ROM BIOS PLUS Replacement Setup Program

IMPORTANT NOTICE

This replacement setup program is provided on an AS-IS basis, and

there is absolutely no warranty, express or implied, as to the

function of this program.

There is the possibility that your system manufacturer may

implement their own non-standard fields in the CMOS RAM that may

not be compatible with this setup program.  In the unlikely event

that this is the case, running this setup program may overwrite

areas in the CMOS RAM used by your system's own non-standard

fields.

Micro Firmware does not provide general computer system or BIOS

technical support.  If you have further questions or problems,

please contact a computer service technician or consultant in your

area.

Introduction

This setup program allows you to set up standard system parameters

that are, for the most part, common to all IBM AT compatible 286,

386, and 486 ISA-based systems.  The standard parameters include

the date and time, floppy drive types, hard drive types, video

adapter type, and memory size.

This does not work on PC/XT Class systems -- there is no such thing as

SETUP on a PC/XT class system.

Extended setup options, such as chip set specific options, are not

included in SETUP.COM because there are no standards for the usage

and format of the extended CMOS RAM locations.  The way these

parameters are stored in the CMOS RAM will vary depending on the

chip set used, and the default parameters and unique features

implemented by the system manufacturer.  Thus, there isn't a

"standard" setup program that allows you to set up extended system

or chip set specific parameters.

If your situation requires the setup of extended parameters, and a

replacement setup program is not available from your system

manufacturer, please call Micro Firmware Technical Support at

405-321-8333, and we'll see what we can do for you.

About the SETUP Diskette

Due to the nature of the high-speed duplicating system used to

produce our distribution diskettes, only the areas of the diskette

that are actually in use have been formatted.  As a result, the

DOS DISKCOPY command, as well as some disk diagnostic utilities,

will report a large number of errors.  This is normal.

However, you will have no problem executing the SETUP program

directly from the disk, or using the COPY command to copy the

SETUP.COM file to another floppy or to your hard drive.

SETUP.COM Operation

Upon running the setup program, you will be alerted if the Power

On Self Test (POST) found any errors at bootup.  Setup will also

advise you of certain other specific configuration errors, too.

At the main setup screen, use the up and down arrows to move

between fields, and you can usually use the left and right arrows

to change the field values. When entering memory sizes or custom

hard drive type data, simply type the numbers in.

The memory size fields will be automatically set to what the Setup

program finds.  This should be the same amount of memory that the

POST finds at bootup.  You should not have to edit the memory size

fields.

The hard drive table built into your BIOS can be displayed by

positioning the cursor on the hard drive type 1 or 2 fields, and

pressing the F1 key twice. Types 1-47 are fixed, and 48-49 are

user definable.

User defined hard drive types are relatively new.  If the

copyright date on your BIOS (look at the Phoenix copyright notice

at bootup) is before 1989, there is a good possibility that your

BIOS does not support user defined hard drive types.

To set up a user defined hard drive type, select type 48 or 49,

then press the down arrow key to move into the actual drive

parameter fields.  At this point, you can type the appropriate

values in.

If you attempt to set up a user defined hard drive type, and your

BIOS does not support it, you will see a "Hard drive configuration

error" message when your system reboots.

Once your setup data has been properly entered and/or edited,

press the ESC key to save the changes and reboot, which makes the

changes active.

Common Questions & Answers

Q. What is CMOS?  How is this different from the BIOS?

A. The CMOS RAM is the low-power, battery-backed RAM in an IBM AT

compatible system that stores system configuration data.  The CMOS

RAM is almost always built-in to the clock chip that keeps the

system date and time.  The setup program is used to edit the data

stored in the CMOS RAM.  While the BIOS reads the values stored in

the CMOS RAM to see how the system is configured, the BIOS and

CMOS RAM are two different things, just the same way the BIOS and

your floppy drive are different.

If the CMOS RAM is not holding the date, time, or other values,

the problem is generally with the battery, or the CMOS RAM/clock

chip itself.

Q. I have more memory than what my system finds at bootup.

Changing the memory size in the setup program has no effect, and

gets changed back to match what the system finds when I reboot.

What's the deal?

A. During a cold boot, the Power On Self Test (POST) will count up

the base and extended memory installed in your system.  Either the

memory is there or it isn't, so the POST doesn't look at what you

have selected in setup to determine how much RAM you have.

However, once the POST determines how much memory you have, the

CMOS RAM setting is checked to verify that they match.  This way,

you are alerted if, for some reason, the amount of memory in your

system appears to have been changed.

If the installed memory does not come up, the problem is most

likely caused by a configuration problem (i.e. jumper and/or

switch settings) on the motherboard or add-in memory card.