Genesis 65XX macro and cross assembler Version 2.2

&  Genesis 65XX resourcer alpha version

(c) 1994-96 by Sven Michael Klose, Bielefeld.

This is free software and a part of the TMA project. Please read COPYING

for details.

FTP:	copei.devcon.net/pub/tma

Emai:	sven@devcon.net

������������������������������������������������������������������������������

� Contents			     �

��������������������������������������

Introduction :

What is Genesis ? ................................................... 1-0

Assembing the first source .......................................... 1-1

Executing the assembled program ..................................... 1-2

Transfer to other platforms ......................................... 1-3

Programming Genesis :

Syntax of the assembler ............................................. 2-0

Syntax of the 65XX mnemonics ........................................ 2-1

Labels .............................................................. 2-2

Mnemonics ........................................................... 2-3

Operands ............................................................ 2-4

Byte/word syntax .................................................... 2-4-1

Arithmetic operations ............................................... 2-4-2

Comments ............................................................ 2-5

Directives .......................................................... 2-6

Error messages ...................................................... 3-0

The 65XX CPU:

The 65XX instruction set in alphabetical order ................. Appendix A

Mnemonics of the undocumented instructions ..................... Appendix B

Opcodes of the undocumented instructions ....................... Appendix C

History of Genesis ............................................. Appendix D

What is Genesis ?

==================

Genesis is a cross assembler for 65x0 CPUs with an amazing turn of speed.

It is able to produce binary and tape files for the "real C64 emulator".

Now it's able to assemble undocumented commands even in the shareware

version.

For modular programming Genesis knows include files.

Macros are available only in the registered version.

1.1 Introduction

========================

Start a trial run with the delivered source file :

GENESIS GAME.A65 GAME.BIN

1.2 How the ���� can I use the code then ?

==========================================================

No problem ! But you should own the *real* C64-Emulator written by

Miha Peternel and it should be installed on your hard disk ...

The PC64 emulator doesn't handle T64 files but you can use raw files.

Type in

GENESIS GAME.A65 c:\C64_EMU\GAME.T64

or

GENESIS GAME.A65 C:\C64_EMU\GAME.BIN for PC64.

Perhaps you'll have to modify the directory name.

Genesis now assembles the code and writes it to disk

as a tape file for the emulator.

Now start the emulator und type LOAD without the RETURN-Key

then press the F9-key and choose

GENESIS 2.2 CREATED TAPE

and

GENESIS 2.2 FILE

Finally type the RUN command. Bad luck ...

1.3 Other ways of using Genesis 6510

=====================================

To transfer your assembled programs easily use one of your

serial or parallel ports of your personal computer.

The handling of Genesis is easy in this way, too.

Type in

GENESIS GAME.A65 COM1

to let Genesis put out the assembled code to COM1.

Anyway, you'll have to link your PC with your good, ol' C64

and you'll have to write a C64 program which receives the data

and writes it to memory or to disk. FIXME.

----------------------------------------------------------------------------

2.2 Syntax of 6510 mnemonics

============================

Genesis accepts more or less standard syntax.

The structure is always the same:

Label:	Mnemonic:  Operand:    Comment:

------------------------------------------------------------------

label	lda	   #$00        ; load accumulator with 0

2.2 Labels

==========

A label always begins with a letter and ends with a space char.

Never use chars which are reserved for operands.

For example :

WriteChar-1

Hello$

A&bs	 will force error messages.

If you can define many labels with ONE adress. For example:

pointer xpos overbyte BYT 0

if you want to save memory.

2.3 Mnemonics

=============

This version accepts standard mnemonics and undocumented mnemonics,

adressing modes, all mnemonics of the 65xx series and it contains a

documentation file with an exact list of their opcodes and timing.

2.4 Operands

============

Following adressing modes are legal :

Adressing mode: 	Example:

---------------------------------

Implied.		ASL

Immediate		LDA #$ff

Zero paged		LDA $12

Zero paged,X		LDA $12,X

Absolute		LDA $4232

Absolute,X		LDA $4232,X

Absolute,Y		LDA $4232,Y

Indirect indexed	LDA ($02,X)

Indexed indirect	LDA ($02),Y

Indirect (for jumps)	(START)

The most of the 65xx-instrucions don't work with all

adressing modes. But Genesis will tell you if the time has come.

2.4.1 Byte/Word Syntax

======================

Genesis accepts following syntax:

Definition		Example:

-----------------------------------

decimal 		LDA #1

hexadecimal		LDA #$a4

binary			AND #%01001001

octal			ORA #!732

character		LDY #"a"

or		LDA #"""

or even 	STA "21" ;the first char defines the high byte

If you only want to use the higher or the lower byte of a word

just type following signs before typing a constant or label :

'>' for the higher byte.

'<' for the lower byte.

Examples:		LDA <screen

STA screenpointer1

LDA >screen

STA screenpointer2

LDY #0

LDA (screenpointer),Y

2.4.2 arithmethic operands ...

==============================

Genesis accepts following operands to make programmer's live easier :

Defintion	 Example

==========================

add		 label + 123

subtract	 label - 123

multiply	 label * 16

divide		 label / 16

AND		 label & $ff

OR		 label | $%1010000000000000

And now a weird example:

label		lda <screenposition+!0345-willy|1211/2*5&%01110110101|2

2.5 Comments

===============

After a comment flag (";") you're able to write any kind of text.

And now for example (yes, I'm a lazy one ):

hallo	sta $d018	; writes accumulator to a VIC register

2.6 Directives

===============

Yes, Genesis understands some :

ORG <word>[,<word>]	; Defines the actual code position.

; The first ORG defines the starting adress

; of the code, the last instructions after

; the last ORG defines the ending position.

; With the second operand you can assemble

; code for another position <word>.

ORG ,<word>		; <- Leaves the real position.

label EQU <word> or	; Defines a label with a word. You can also use

label =   <word>	; it for immediate adressed instructions if

; it has got byte size.

; flag=255

; LDA #flag

BYT <byte1>,<byte2>,... ; Writes byte(s) to actual position.

WOR <word1>,<word2>,... ; Does the same with words.

ASC "Text"		; Does the same with ASCII texts

ASC "Text",<value>	; You can only use byte values with the ASC-Kommand

LST label		; Prints definition of label to screen at

; assembly.

SYS label		; Creates a the line "0 SYS <label's def> for a

; CBM-BASIC program.

; This command also includes an ORG $801.

; The following code starts at $80f

EXT "filename.asm"	; Includes A65-files.

; EXT commands in include-files work, too.

EXB filename.bin	; Includes binary files to your code.

Following directives only exist in the full version :

macroname MAC BYT,WOR,WOR   ; Defines a macro

.EM			    ;

label MAC macroname	    ; Calls macro

.IF/ELS/END/NOT/OR/AND	    ; Comparisons for macro structure

; at assembly only.

3 Error messages

======================

... explain themselves.

The full version inserts error messages to your source code and erases

them if the errors have been removed.

This shareware version just prints error messages to your screen.

Appendix A

==========

6510-CPU instruction set in alphabetic order

---------------------------------------------

ADC	Add memory to accumulator with carry flag

AND	"AND" memory with accumulator

ASL	Shift left one bit (memory or accumulator)

BCC	Branch on carry clear

BCS	Branch on carry set

BEQ	Branch on result zero

BIT	Test bits in memory with accumulator

BMI	Branch on result minus

BNE	Branch on result not uero

BPL	Branch on result plus

BRK	Force break

BVC	Branch on overflow clear

BVS	Branch on overflow set

CLC	Clear carry flag

CLD	Clear decimal mode

CLI	Clear interrupt disable bit

CLV	Clear overflow flag

CMP	Compare memory and accumulator

CPX	Compare memory and index X

CPY	Compare memory and index Y

DEC	Decrement memory or accu by one

DEX	Decrement index X by one

DEY	Decrement index Y by one

EOR	"exclusive-OR" memory with accumulator

INC	Increment accu or memory by one

INX	Increment index X by one

INY	Increment index Y by one

JMP	Jump to new location

JSR	Jump to new location saving return adress

LDA	Load accu with memory

LDX	Load index X with memory

LDY	Load index Y with memory

LSR	Shift right one bit (memory or accumulator)

NOP	No operation

ORA	"OR" memory with accumulator

PHA	Push accumulator on stack

PHP	Push processor status on stack

PLA	Pull accumulator from stack

PLP	Pull processor status from stack

ROL	Rotate one bit left (memory or accumulator)

ROR	Rotate one bit right (memory or accumulator)

RTI	Return from interrupt

RTS	Return from subroutine

SBC	Subtract memory from accumulator with carry flag

SEC	Set carry flag

SED	Set decimal mode

SEI	Set interrupt disable flag

STA	Store accumulator in memory

STX	Store index X in memory

STY	Store index Y in memory

TAX	Transfer accumulator to index X

TAY	Transfer accumulator to index Y

TSX	Transfer stack pointer to index X

TXA	Transfer Index X to accumulator

TXS	Transfer Index X to stack pointer

TYA	Transfer index Y to accumulator

Appendix B

==========

Instruction adressing modes and related execution times in clock cycles :

----------------------------------------------------------------------------

Ac/Im	= accu/implied

It	= immediate

ZP	= zero page

ZX	= zero page indexed with X register

ZY	= zero page indexed with Y register

Ab	= absolute

AX	= absolute indexed with X register

AY	= absolute indexed with Y register

In	= indirect

IX	= indexed indirect with X register

IY	= indirect indexed with Y register

Ac

Instruction	Im It ZP ZX ZY Ab AX AY In IX IY

------------------------------------------------------------------------

ADC		.  2  3  4  .  4  4* 4* .  6  5*

AND		.  2  3  4  .  4  4* 4* .  6  5*

ASL		2  .  5  6  .  6  7  .	.  .  .

BCC		.  .  .  .  .  .  .  .	2# .  .

BCS		.  .  .  .  .  .  .  .	2# .  .

BEQ		.  .  .  .  .  .  .  .	2# .  .

BIT		.  .  3  .  .  4  .  .	.  .  .

BMI		.  .  .  .  .  .  .  .	2# .  .

BNE		.  .  .  .  .  .  .  .	2# .  .

BPL		.  .  .  .  .  .  .  .	2# .  .

BRK		.  .  .  .  .  .  .  .	.  .  .

BVC		.  .  .  .  .  .  .  .	2# .  .

BVS		.  .  .  .  .  .  .  .	2# .  .

CLC		2  .  .  .  .  .  .  .	.  .  .

CLD		2  .  .  .  .  .  .  .	.  .  .

CLI		2  .  .  .  .  .  .  .	.  .  .

CLV		2  .  .  .  .  .  .  .	.  .  .

* = Add one cycle if indexing across page boundary.

# = Add one cycle if branch is taken and

add one additional if branching operation crosses page boundary.

Ac

Instruction	Im It ZP ZX ZY Ab AX AY In IX IY

------------------------------------------------------------------------

CMP		.  2  3  4  .  4  4* 4* .  .  5*

CPX		.  2  3  .  .  4  .  .	.  .  .

CPY		.  2  3  .  .  4  .  .	.  .  .

DEC		.  .  5  6  .  6  7  .	.  .  .

DEX		2  .  .  .  .  .  .  .	.  .  .

DEY		2  .  .  .  .  .  .  .	.  .  .

EOR		.  2  3  4  .  4  4* 4* .  6  5*

INC		.  .  5  6  .  6  7  .	.  .  .

INX		2  .  .  .  .  .  .  .	.  .  .

INY		2  .  .  .  .  .  .  .	.  .  .

JMP		.  .  .  .  .  3  .  .	5  .  .

JSR		.  .  .  .  .  6  .  .	.  .  .

LDA		.  2  3  4  .  4  4* 4* .  6  5*

LDX		.  2  3  .  4  4  .  4* .  .  .

LDY		.  2  3  4  .  4  4* .	.  .  .

LSR		2  .  5  6  .  6  7  .	.  .  .

NOP		2  .  .  .  .  .  .  .	.  .  .

ORA		.  2  3  4  .  4  4* 4* .  6  5*

* = Add one cycle if indexing across page boundary.

Ac

Instruction	Im It ZP ZX ZY Ab AX AY In IX IY

------------------------------------------------------------------------

PHA		3  .  .  .  .  .  .  .	.  .  .

PHP		3  .  .  .  .  .  .  .	.  .  .

PLA		4  .  .  .  .  .  .  .	.  .  .

PLP		4  .  .  .  .  .  .  .	.  .  .

ROL		2  .  5  6  .  6  7  .	.  .  .

ROR		2  .  5  6  .  6  7  .	.  .  .

RTI		6  .  .  .  .  .  .  .	.  .  .

RTS		6  .  .  .  .  .  .  .	.  .  .

SBC		.  2  3  4  .  4  4* 4* .  6  5*

SEC		2  .  .  .  .  .  .  .	.  .  .

SED		2  .  .  .  .  .  .  .	.  .  .

SEI		2  .  .  .  .  .  .  .	.  .  .

STA		.  .  3  4  .  4  5  5	.  6  6

STX		.  .  3  .  4  4  .  .	.  .  .

STY		.  .  3  4  .  4  .  .	.  .  .

* = Add one cycle if indexing across page boundary.

Ac

Instruction	Im It ZP ZX ZY Ab AX AY In IX IY

------------------------------------------------------------------------

TAX		2  .  .  .  .  .  .  .	.  .  .

TAY		2  .  .  .  .  .  .  .	.  .  .

TSX		2  .  .  .  .  .  .  .	.  .  .

TXA		2  .  .  .  .  .  .  .	.  .  .

TXS		2  .  .  .  .  .  .  .	.  .  .

TYA		2  .  .  .  .  .  .  .	.  .  .

Appendix C

===========

Undocumented 6502 commands in alphabetical order

------------------------------------------------

Note:	Only the full version of Genesis knows undocumented mnemonics.

They can be very helpful. For example "LAX"

AAX	ANDs X with A and stores it to memory

ARR	ANDs accu with memory and rotates it right

ASR	ANDs accu with memory and shifts it right

ATS	ANDs accu with memory and sets carry flag if it's signed

AX2	ANDs A and X and $21 and store the result to memory

AY2	ANDs A and Y and $21 and store the result to memory

DCP	DEC and CMP with given adressing mode

HLT	Halts system totally

ISB	INC and SBC with given adressing mode

LAX	Load A and X register with memory

MAX	ANDs memory with X register and stores bits 1-7 to accu

Bit 0 of A will be the result of A0 AND X0 AND M0

RLA	ROL and AND with given adressing mode

SLO	ASL and ORA with given adressing mode

SPM	Store result of (SP and M) to A, X and SP

SRE	LSR and EOR with given adressing mode

SX0	Stores X register to memory if it's zero

RRA	ROR and ADC with given adressing mode

XAM	Store result of (X and A) to memory and X

Appendix D

===========

Undocumented 6510 commands in order of values

----------------------------------------------

Notes:	In the listing below there are some shortcuts used :

A = Accumulator

, X = Index register X , Y = Index register Y , SP = Stackpointer

, M = byte in Memory , -> = is Stored in.

The flag settings are, unless other specified,

calculated in the same way as the intructions they consist of.

value	mnemonic Adressing mode  effect

--------------------------------------------

02	HLT	 -		 Total HALT

03	SLO	 (Ind,X)	 ASL,ORA

04	NOP	 Zeropage	 NOP

07	SLO	 Zeropage	 ASL,ORA

0B	ATS	 Immediate	 AND #$xx,ASL A with only setting flags

0C	NOP	 Absolute	 NOP

0F	SLO	 Absolute	 ASL,ORA

12	HLT	 -		 Total HALT

13	SLO	 (Ind),Y	 ASL,ORA

14	NOP	 Zeropage	 NOP

17	SLO	 Zeropage,X	 ASL,ORA

1A	NOP	 Implied	 NOP

1B	SLO	 Absolute,Y	 ASL,ORA

1C	NOP	 Absolute	 NOP

1F	SLO	 Absolute,X	 ASL,ORA

22	HLT	 -		 Total HALT

23	RLA	 (Ind,X)	 ROL,AND

27	RLA	 Zeropage	 ROL,AND

2B	ATS	 Immediate	 AND #$xx,ROL A with only setting flags

2F	RLA	 Absolute	 ROL,AND

32	HLT	 -		 Total HALT

33	RLA	 (Ind),Y	 ROL,AND

34	NOP	 Zeropage	 NOP

37	RLA	 Zeropage,X	 ROL,AND

3A	NOP	 Implied	 NOP

3B	RLA	 Absolute,Y	 ROL,AND

3C	NOP	 Absolute	 NOP

3F	RLA	 Absolute,X	 ROL,AND

42	HLT	 -		 Total HALT

43	SRE	 (Ind,X)	 LSR,EOR

44	NOP	 Zeropage	 NOP

47	SRE	 Zeropage	 LSR,EOR

4B	ASR	 Immediate	 AND #$xx,LSR A

4F	SRE	 Absolute	 LSR,EOR

52	HLT	 -		 Total HALT

53	SRE	 (Ind,Y)	 LSR,EOR

54	NOP	 Zeropage	 NOP

57	SRE	 Zeropage,X	 LSR,EOR

5A	NOP	 Implied	 NOP

5B	SRE	 Absolute,Y	 LSR,EOR

5C	NOP	 Absolute	 NOP

5F	SRE	 Absolute,X	 LSR,EOR

62	HLT	 -		 Total HALT

63	RRA	 (Ind,X)	 ROR,ADC

64	NOP	 Zeropage	 NOP

67	RRA	 Zeropage	 ROR,ADC

6B	ARR	 Immediate	 AND #$xx,ROR A

6F	RRA	 Absolute	 ROR,ADC

72	HLT	 -		 Total HALT

73	RRA	 (Ind),Y	 ROR,ADC

74	NOP	 Zeropage	 NOP

77	RRA	 Zeropage,X	 ROR,ADC

7A	NOP	 Implied	 NOP

7B	RRA	 Absolute,Y	 ROR,ADC

7C	NOP	 Absolute	 NOP

7F	RRA	 Absolute,X	 ROR,ADC

80	NOP	 Zeropage	 NOP

82	NOP	 Zeropage	 NOP

83	SX0	 (Ind,X)	 0->M if X=0 , No flags set

87	AAX	 Zeropage	 A and X -> M , No flags set

8B	MAX	 Immediate	 M and X -> A (bits 7-1)

M0 and X0 and A0 -> A0, No flags set

8F	AAX	 Absolute	 A and X -> M , No flags set

92	HLT	 -		 Total HALT

93	AX2	 (Ind),Y	 A and X and $21 -> M , No flags set

97	AAX	 Zeropage,Y	 A and X -> M , No flags set

9B	AX2	 Absolute,Y	 A and X and $21 -> M , A and X -> SP, No flags set

9C	AY2	 Absolute,X	 A and Y and $21 -> M , No flags set

9E	AY2	 Absolute,Y	 A and X and $21 -> M , No flags set

9F	AX2	 Absolute,Y	 A and X and $21 -> M , No flags set

A3	LAX	 (Ind,X)	 LDA,LDX

A7	LAX	 Zeropage	 LDA,LDX

AB	LAX	 Immediate	 LDA,LDX

AF	LAX	 Absolute	 LDA,LDX

B2	HLT	 -		 Total HALT

B3	LAX	 (Ind),Y	 LDA,LDX

B7	LAX	 Zeropage,Y	 LDA,LDX

BB	SPM	 Absolute,Y	 SP and M -> A,X,SP

BF	LAX	 Absolute,Y	 LDA,LDX

C2	NOP	 Zeropage	 NOP

C3	DCP	 (Ind,X)	 DEC,CMP

C7	DCP	 Zeropage	 DEC,CMP

CB	XAM	 Immediate	 (X and A) - M -> X

CF	DCP	 Absolute	 DEC,CMP

D2	HLT	 -		 Total HALT

D3	DCP	 (Ind),Y	 DEC,CMP

D4	NOP	 Zeropage	 NOP

D7	DCP	 Zeropage,X	 DEC,CMP

DA	NOP	 Implied	 NOP

DB	DCP	 Absolute,Y	 DEC,CMP

DC	NOP	 Absolute	 NOP

DF	DCP	 Absolute,X	 DEC,CMP

E2	NOP	 Zeropage	 NOP

E3	ISB	 (Ind,X)	 INC,SBC

E7	ISB	 Zeropage	 INC,SBC

EB	SBC	 Immediate	 SBC #$xx

EF	ISB	 Absolute	 INC,SBC

F2	HLT	 -		 Total HALT

F3	ISB	 (Ind),Y	 INC,SBC

F4	NOP	 Zeropage	 NOP

F7	ISB	 Zeropage,X	 INC,SBC

FA	NOP	 Implied	 NOP

FB	ISB	 Absolute,Y	 INC,SBC

FC	NOP	 Absolute	 NOP

FF	ISB	 Absolute,X	 INC,SBC

Appendix E

===========

History of Genesis 6510

------------------------

v1.0	-	First release with include files

v1.1/1.2-	Some internal errors have been removed.

v2.0	-	Undocumented instructions of the 6510 CPU and the

EXB directive are now included.

The ASC and BYT directive have been extended to do the same.

Genesis also allows you to use source codes bigger than

64k.

v2.1	-	Bugfixed and faster but quite the same as V2.0

v2.2	-	The ORG directive has been extended. Now you can define

and index to the adresses. Very useful.