LSppp is a small MSDOS PPP packet driver.  It is compatible with the

needs of WATTCP and trumpet stacks.  The resident size is approximately

28K.

Note that the undocumented /l option has been changed to the

undocumented /z option.  Use of the new /l option will in general cause

internet connections to not work.

LSppp loads as a class one ethernet driver and handles all

aspects of the connection from dialing onward.  Optionally, an external

dialer may be used to dial an ISP prior to running LSppp; it will

detect the carrier and go immediately into internet communications

setup.

LSppp is capable of auto-configuring WATTCP through the bootp mechanism

and will do so automatically.  However, for this to work properly LSppp

will have to be supplied with name server information.  LSppp will

attempt to automatically load name server information from the ISP

if none is specified on the command line, however, this is an optional

feature your ISP may not support.  To see if it is supported run LSppp

without specifying the name server switch (/N) and see if it places

DNS addresses in the IP-UP.bat file

LSppp does not use configuration files; all parameters are placed on

the command line or optionally in the LSPPP environment variable.

LSPPP will search the environment variable for valid command switches

first, then modify this information with command switches specified on

the command line.  Configuration options include user name and

password, number to dial, nameserver information, and various options

to set up a communications port to use or customize the timeout

mechanisms.

The current version does not have a lot of backwards compatability for

old versions of the protocols.  If you have problems running it use the

undocumented /z switch and send the resulting LSPPP.LOG file to the

author for analysis.  Note that to get a log file you MUST use the /z

switch; the /l switch is now used for local mode

Van jacobsen compression and the CHAP protocol have recently been added

to LSPPP.  Other less used protocols (including MSCHAP) are not

currently supported.

When dialing the modem sends the following configuration string:

ATZ

Also if /dm:# is specified the modem will be told to mute the speaker.

At any time before LSICQ goes resident, you may press 'esc' to end the

session and hang up.

Configuration options are as follows:

/a:$	set async map

set an asynchronous control map.  Characters in the map will be

treated as purely control characters and will be escaped during

transmission.  For example if the modem uses XON and XOFF you

would specify a map of 0x000A0000.

The default is a map of zero, meaning no characters are escaped.

/b:$    Comm port

Set a base i/o port.  For example /b03f8

default: /b:03f8

/d[r][m][f][l][e]:* Dial [redial] phone number

Set the phone number to dial.  The redial option is not

supported yet.  The 'm' option tells lsppp to mute the speaker.

The 'f' option forces it to dial even when it thinks it is

already connected (useful with winmodems).  The 'l' option

causes lsppp to initially attempt a terminal-mode login before

attempting to connect with the PPP protocol.  The 'e' option

causes lsppp to dial with 'ATX3D' instead of 'ATD', which

should be useful in Europe where no dial tone exists.

Example:  /d:5551214

Example:  /d:P5551214 this does pulse dialing

default: LSPPP does not dial

/h or /?

Displays the help text

/i      Comm IRQ (0-15)

Set the interrupt vector to use

example : /i:4

default:  /i:4

/l	Local mode

Forces LSICQ to not dial (local mode connection)

Also turns off use of modem control signals

/n:#    Comm port

Set the com port (1-4).  Automatically sets the /b and /i

options based on the setting.

example: /n:1

default: /n:1

/u      Unload TSR

unloads the tsr

/x	xon/xoff

use the xon/xoff protocol instead of modem control signals

/A:#,#    PAP timeout and count

Set the timeout and count governing password authentication.

The timeout is in seconds and is the time between retries; when

count retries have been done LSPPP will give up.  default:

/A:3,10

/B:#    Baud rate

Set the baud rate

example: /B:57600

default: /B:19200

/D:#,#,#  Dial timeout & count & delay

Set the timeout and retries for dialing.  This is the time

LSPPP will wait for a connection (in seconds) and the number of

retries to make before LSPPP gives up entirely.  The third

parameter is optional and is the number of seconds after

dialing before it tries to connect to the ISP.

default:  /D:45,1,1

/F:*	UART Fifo buffering

Set the buffering level of the Uart's receive fifo (if

present).  Valid values are 1,4,8, and 14

default: /F:1

/I:#,#  IPCP timeout & count

Set the timeout and count governing IP configuration.

The timeout is in seconds and is the time between retries; when

count retries have been done LSPPP will give up.

default: /I:3,10

/L:#,#  LCP timeout & count

Set the timeout and count governing initial link configuration.

The timeout is in seconds and is the time between retries; when

count retries have been done LSPPP will give up.

default: /L:3,10

/M:*    Modem init string

Specify a string to use to initialize the modem when dialing.

A typical example is AT&F to initialize defaults.  Note that

the dialer will always send ATZ prior to sending the initialize

string, which resets the modem.  Also the 'm' parameter on the

dialer can be used to automatically send ATM0 (mutes the modem)

in addition to this initialize string

default:  AT&F

/N:@[,@]        DNS address

Set the DNS name server address[s].  First address is primary

and second address is secondary.  If no name servers are

specified LSPPP will attempt to auto-configure them from the

server.

example:  /N:198.77.33.2,198.77.33.1

default: /N:0.0.0.0,0.0.0.0 (autoconfigure)

/P:*    passwd

Set the login password

example: /P:truely

default: no login password

/U:*    User name

Set the login user name

example: /U:dinosaur

default: user name

/V:$    Packet driver vector

Set the packet driver vector to use

example: /V:60

default: LSPPP searches for a free vector

An example of a valid command line is:

lsppp /dm:5551214 /n:3 /B:57600 /N:101.202.101.4 /U:myname /P:mypass

which uses COM3 to log into an ISP at 57600 BPS with account 'myname'

and password 'mypass'.  A nameserver is specified and the speaker will

be muted.

If we had performed the following to set the LSPPP environment

variable:

set LSPPP=/n:3 /B:57600 /U:myname /P:mypass

Then typing

lsppp /dm:5551214 /N:101.202.101.4

would have the same effect as the first commmand line since LSPPP will

read the environment varible before reading the command line.  Of

course you can put the number to dial and nameserver in the environment

too.

LSPPP Copyright (c) LADSoft

November, 2000

David Lindauer

camille@bluegrass.net