Sydex

RainDOS

A DOS Device Driver for DEC Rainbow MS-DOS(tm) Diskettes

Sydex

P.O. Box 5700

Eugene, OR 97405

(503) 683-6033

RainDOS

A Device Driver for DEC Rainbow MS-DOS Diskettes

Copyright 1990-1995, Sydex

All Rights Reserved

Version 1.22 May, 1993

Sydex

P.O. Box 5700

Eugene, OR  97405

(503) 683-6033

What is RainDOS?

At approximately the same time as the introduction of the IBM Per-

sonal Computer, Digital Equipment Corporation was promoting their

own dual-CPU personal computer, the Rainbow.  In addition to run-

ning 8- and 16-bit CP/M applications,  these systems also  ran  a

special versions of Microsoft's MS-DOS(tm) software.  In spite of

being  able  to  run  some  common  MS-DOS  applications,  from a

hardware standpoint the Rainbow is quite different from  the  IBM

PC and its clones.  The standard 5.25" diskette used on the Rain-

bow  was an 80-track single-sided double-density format,  as con-

trasted with "standard" MS-DOS 5.25" DSDD diskettes which  are  a

40 track double-sided format.

RainDOS  is  an  installable device driver which support reading,

writing and formatting of MS-DOS diskettes from the Rainbow on PC

XT- or  AT-compatible  machines.    All  diskette  functions  and

utilities FORMAT, DISKCOPY and DISKCOMP are supported.

Page 1

Configuration Requirements

RainDOS  can  be  installed  on  a  PC  AT-,  XT- or IBM Personal

System/2-compatible computer.   Computers such as the Tandy  2000

or  the  Zenith  Z-150 are not suitable because of their relative

incompatibility.   In addition,  the PC must be equipped  with  a

3.5"  720K  or 5.25" 1.2 Mbyte diskette drive and a diskette con-

troller that supports the 1.2 Mbyte high-density format.

PC-DOS or MS-DOS Version 3.3 or later must be  installed  as  the

PC's operating system.   While it may be possible to operate Rain-

DOS under Novell's DR-DOS,  The Software Link's PC-MOS,  or IBM's

OS/2 (DOS sessions only),  these operating systems have not  been

tested for their compatibility with RainDOS and operation of Rain-

DOS  under  non-Microsoft  operating  systems is not warranted by

Sydex.

Installing and Using RainDOS

Installation of RainDOS is straightforward.  First, find the file

CONFIG.SYS in the root directory of the disk  used  to  boot  (or

load) DOS.  In most cases, this will be the root directory of the

first hard disk, usually called drive C:.

Copy the file RAINDOS.SYS from the distribution diskette into the

root directory of the boot drive.   For example, if the distribu-

tion diskette is inserted into diskette drive  A:  and  the  boot

drive is C:, the following DOS command could be used:

COPY A:RAINDOS.SYS C:\

DOS should respond with the message:

1 file(s) copied.

The  file  named  CONFIG.SYS  on  the boot drive next needs to be

modified to inform DOS that RAINDOS.SYS is to be  installed  when

DOS  is booted.   Use an ASCII text editor such as DOS's EDLIN or

DOS 5.0 EDIT to do this.   Do not use the document mode of a word

processing package,  such as Word Perfect to do this.  At or near

the last line in the CONFIG.SYS file, insert a line of the follow-

ing form:

DEVICE=C:\RAINDOS.SYS

Page 2

The above line assumes that the file RAINDOS.SYS has been  copied

to the root directory on drive C: as described above.

Finally, re-boot the PC using the Ctrl-Alt-Del keys or by turning

the PC power off, then on again.

After  DOS  has  loaded,  a  message similar to the following one

should appear:

DEC Rainbow 5.25" diskette driver V1.22

Copyright 1995, Sydex, Inc.  All Rights Reserved.

DEC Rainbow DOS diskette unit 0 is accessible as drive K:

This message indicates that RainDOS has been  installed  success-

fully.   The diskette unit number specifies the physical diskette

drive that will be used to access DEC Rainbow  diskettes.    Unit

number  0 is the first diskette drive on a system (the A: drive),

unit 1 is the second,  and so forth.   The drive letter given  in

the  message  indicates  what drive letter must be used to access

DEC Rainbow diskettes placed in the diskette drive.    Thus,  the

physical diskette drive that is used to process Rainbow diskettes

is accessed by two different drive letters.  The first drive let-

ter is used to access DOS diskettes in the drive; the second, as-

signed by RainDOS is used to handle DEC Rainbow diskettes.

This installation procedure should not need to be repeated unless

the  RAINDOS.SYS  file is erased or corrupted by an operator mis-

take or system failure.

Installation Problems

There are a few situations where an error message will appear in-

stead of the usual installation message.

The following error message is displayed when RainDOS cannot  lo-

cate a 5.25" high-density drive on the PC:

Could  not  find a 1.2M 5.25" drive.   Use the /D: switch to

specify one.

Page 3

This message may appear on systems where the  5.25"  high-density

drive has been installed as an add-on or external drive.   It may

also be displayed if a high-density drive has been  added  to  an

XT-class  system.   It will be necessary to specify the BIOS unit

number of the drive explicitly in the CONFIG.SYS file.   This  is

done by adding the /D:unit switch to the line in CONFIG.SYS which

specifies  the  location  of  RAINDOS.SYS.   Thus,  to explicitly

specify drive unit 2 (probably the third diskette drive), the fol-

lowing statement would be used in CONFIG.SYS:

DEVICE=C:\RAINDOS.SYS /D:2

External or add-on diskette drives are accessed by BIOS unit num-

bers that are largely determined by their manufacturers.   If you

experience difficulty in determining the BIOS unit number that is

associated with an external drive, you should consult the manufac-

turer of the drive for assistance.

If  extra characters or other typographical errors are present in

the CONFIG.SYS file statement for RainDOS,  the following message

will be displayed:

Error in device driver statement.

The error must be corrected and the PC re-booted.

Operating Considerations

In  general,  operation  of  the  RainDOS-managed  diskette drive

should  be  identical  to  that  of  any  other  diskette  drive.

However, there are some possibly unexpected consequences of using

DEC Rainbow diskettes in a PC.

DEC Rainbow hardware is not compatible with PC hardware.  The im-

plication  of  this  is  that  only  the  simpler DEC Rainbow DOS

programs will run successfully on a PC.    Programs  that  access

BIOS  or  hardware interfaces on the Rainbow will not run on a PC

and may produce unpredictable results ranging from system  lockup

to destruction of data.

Some  low-level  PC  data-recovery  utilities  such as the Norton

Utilities may not process DEC Rainbow diskettes correctly.  These

programs make assumptions about  diskette  format  that  are  not

valid for DEC Rainbow diskettes.

Page 4

When  creating  DEC  Rainbow diskettes,  use 5.25" double-density

(DSDD) media.  Use of high-density media (DSHD) will not be reli-

able.

In Conclusion

If you have any questions about this or any other Sydex  product,

please feel free to contact us.   We are available from 9 AM to 5

PM, Pacific Time, Monday through Friday.  You can reach us at:

(503) 683-6033 (Voice)

(503) 683-1622 (FAX)

(503) 683-1385 (BBS)

----------------------------------------------------------

MS-DOS and Windows  are  trademarks  of  Microsoft  Corporation.

WordPerfect is a trademark of WordPerfect Corporation.  IBM, Per-

sonal System/2, and OS/2 are trademarks of International Business

Machines Corporation.   DR-DOS is a trademark of Novell.   PC-MOS

is a trademark of The Software Link.

Page 5