FREEWARE

Courtesy of KD9JQ

PLL2 Version 2.00

USER'S  GUIDE

INTRODUCTION

PLL2 Version 2.00 provides the designer with an interactive tool in

developing 2nd Order Phase Locked Loops.  Design parameters are now

accessible to the user in making design decisions which previously

involved long, tedious calculations, subject to errors or required

expensive, non-user friendly programs.

PLL2 is an Engineering Tool used to calculate Loop Filter component

values through use of previously developed equations.  A perfect

integrator is assumed during this process.  PLL2 should not be

considered to be a "Spice" like simulation program.

Through the use of a BODE Plot and subsequent calculations of pertinent

design data the engineer is assured a high probability of certainty that

his/her design will provide the desired performance.

A Spice or other simulation program can then be run on the PLL circuit

using these calculated loop components and user selected component

models to verify that use of these parts will meet design criteria.

Verification of actual circuit performance is left up to the user.

MENU SELECTION

(1) New Design    "For 1st time designs"

(2) Old Design    "Input Component & System Constants of a

previous design or load a previous design"

(3) Circuit       "Displays Component Assignment for Active

and Passive 2nd Order PLL"

NEW DESIGN

[Use Exponential Values for Numeric Entries "100E-12, 100E3, ETC."]

If you picked (1) NEW DESIGN then the INPUT DESIGN PARAMETERS Window

will pop up requesting input data.  Pop-up Help screens can be pulled

up by entering H.  Press "F1" for default test circuit values.

Page 2

1ST ENTRY - Enter desired 3dB Loop Bandwidth (Hz). Do not confuse this

with the Natural Frequency.

2ND ENTRY - Enter the desired Damping Factor, .707 typical, .5 to 1.4

useable depending on design criteria.

3RD ENTRY - Enter the VCO Tuning Sensitivity in HZ/VOLT.

The program automatically multiplies this by 2*PI to

provide the correct RAD/SEC/VOLT units.

4TH ENTRY - Enter the Phase/Frequency Detector Constant in VOLTS/RAD

After entering your value, a window will popup requesting

selection of a generic detector you will be using in your

design.  Enter the number of the detector.  This will now

provide a more accurate value for the Lockin Range, Time,

Pullin Range, Time, and Tracking Range.  Default = 1.

5TH ENTRY - Enter any optional DC Gain KA (Voltage Ratio) between the

Detector and the Integrator or Integrator and the VCO. Use

care not to saturate any following stages.  Any losses can

be entered by values less than one.  Normally use a value

of one for new designs.

6TH ENTRY - Enter Frequency Division "N".  N = VCO Fo / Ref Frequency.

7TH ENTRY - Enter OP-Amp Open Loop Gain KO as specified in data sheet.

KO entry is omitted for Passive (KO set to 1).

8TH ENTRY - Enter Reference Frequency (Hz) being used.

A new window will now appear displaying the Filter Time Constants and

desired Damping Factor.  A Warning message will appear if the Loop Gain

is inadequate at this point.  The user can press F10 to adjust KA

to bring the design into range (more than one attempt may be required)

or <CR> to bypass.  If you use the "bypass" option to continue the

results may be incorrect or non-exsistent.

Once the correct Gain range has been found enter a standard value for

R2 (Ohms).  Default values based on 3dB BW can be selected by F1.

Another Window will then pop up displaying calculated values for the

Loop Filter.  If these values are close to standard values press (2).

If not, press (1) and enter a new R2.  Higher values of R2 will give

lower C1 values, while lower R2 values will increase C1 values.

Once you have pressed (2) you will be asked to enter standard values

for R1, C1, and R2.   Units are Ohms and Farads.

The designer can simply hit the Carriage Return when this window pops

up if the calculated values are to be used.

The BODE PLOT will then be displayed.

Page 3

OLD DESIGN

If you picked (2) OLD DESIGN then the INPUT DESIGN PARAMETERS Window

will pop up requesting if you wish Manual Data Entry of an previous

design or Select a previous design file.

If the "Manual Entry" is selected a window will pop up requesting

you to input data for R1, C1, R2 and some of the system parameters

requested in the "NEW DESIGN" window.

If the "Select Entry" is selected then a previous design file can

picked by entering the name ofthe file.  The .PL2 extension is assumed.

The BODE PLOT will then be displayed.

The Warning Messages are initially disabled for old files until

re-activated when entering into the Interactive Mode [F3].

CIRCUIT

A schematic representation of the Active and Passive PLL component

assignments are displayed.

BODE PLOT

The BODE Plot provides the designer with useful visual input for

determining design criteria.  Loop Gain range detection to +/- 1%

is in effect and will prompt you for Auto KA or <CR>.

The "SHIFT + F1" key will cause a B&W Screen Dump to printer.

See DISPLAY.CFG configuration file for setting up your printer.

Marker f1 should be < f2 and marker fr >> f3 > fn.  Preferably make f1

at least a decade lower than f2.  Where fr = reference, fn = Natural

Frequency.

Any positions outside of these markers will flag the designer with a

Warning Message and a general solution to take.

The Loop Filter response (White)

Note f1 and f2 markers.

The Closed Loop Response (Blue)

Observe the Closed Loop Gain relative to unity (0dB) at Marker fr.

This will indicate the relative Contribution or Suppression of Noise

to the system caused by any N Divisor taken at the Reference Frequency.

A negative value indicates added suppression, while positive indicates

added noise.

Observe the value of VCO Error Response (Green) below Marker f3

relative to unity (0dB).  This is the Low Frequency Tracking

capabilities of the Loop due to the VCO Integration.

TRANSIENT ANALYSIS RESPONSE

Pressing the TAB key will display the Normalized Frequency and Phase

Step Transient Responses for the actual Damping Factor versus a range

of 0 to 10 Time Constants.  The Frequency Step will response is shown

as yellow, while the Phase Step is shown as green.  Generally takes

5 Time Constants for the response to settle to the 5% point.

Page 4

DESIGN PARAMETER DOCUMENTATION WINDOW

The DOCUMENTATION WINDOW can be accessed by Pressing the Return Key.

A compilation of current design data will then be displayed.

The designer then has the option of printing out the documentation

(SHIFT + F1), starting a new design (F2), modifing current values (F3),

show the BODE Plot again (F4), or saving the design to a file (F5).

Pressing "Q" will end the program without any file save.

Pressing "H" will popup a window defining the display parameters.

The NEW PARAMETERS selection (F3) will pop up a Window of current

design parameters.  To change values simply enter the selection and then

enter the new value.  Pressing the Return Key enters the new value.

To Recalculate the loop filter component values press the "R" key.

To display the new BODE Plot to see the affects of any modifications

press the "P" Key.

SYMBOL DEFINITIONS

R1   Input Gain Set Resistor to Inverting Input of OP-Amp

(Integrator Stage).  Determines F1 in conjunction with C1 and the

DC Gain of OP-Amp.

C1   Set F1 Pole Frequency in conjunction with R1.  Connected from

Inverting Input of OP-Amp (Integrator Stage) to of R2.

R2   Sets F2 Zero in conjunction with C1.  Series connection with C1 and

OP-Amp (Integrator Stage) output.

T1   Time Constant of R1 * C1       (Active)

Time Constant of (R1+R2) * C1  (Passive)

T2   Time Constant of R2 * C1

F1   F1 = 1 / (KO * 2 * PI * T1)  Pole  (Hz)

F2   F2 = 1 / (2 * PI * T2)  Zero  (Hz)

DF   DF = T2 * WN / 2  Damping Factor    (Active)

DF = SQR(1/(T1*K)) * (1 +(T2*K))/2  (Passive)

FN   WN = SQR(K / T1)  Loop Natural Frequency (Hz)

FN = WN/(2*PI)  (Hz)

FT   FT = KL * 1.8 * WN * (DF + 1) / (2 * PI)   Tracking Frequency (Hz)

See Table 1 under PHASE DETECTOR KD for KL values

NOISE BW  ~= .25 * KF * (1 + (A2 / KT))    (Active)

~= .25 * KF * (KT + A2) / KT     (Passive)

where A2 = (1/T2) - (1/T1)

KF = K * T2 / T1

KT = KF + (1/T1)

Page 5

LOCKUP FREQ FL is frequency delta less than the 3dB BW.  Loop will lock

up without slipping cycles in this range.

FL ~= KL * 2 * DF * FN

LOCKUP TIME  TL Time required for lock without slippage is ~= 1 / WN

PULL IN LIMIT FP   Frequency delta from lock that will still lockup but

with slippage.

FP ~= KL*SQR(4*DF*WN*K-2*WN^2) / (2*PI)  (Hz)    (Detectors 1,2,3,4)

FP ~= KL*PI*WN*EXP(F(DF))/(2*PI)                 (Detectors 5 & 6)

F(DF)=XDF*ATN(1/XDF)

where XDF=DF/SQR(DF^2-1)  for DF > 1

where XDF=DF/SQR(1-DF^2)  for DF < 1

where XDF=0               for DF = 1

PULL IN TIME TP ~=  Time to lockup with slippage after an initial

frequency error equal to FP.  Let WP=2*PI*FP

TP ~=   WP^2/(2*PI^2*DF*WN^3)  (Detectors 1,2,3,5,6)

TP ~= 4*WP^2/(2*PI^2*DF*WN^3)  (Detector 4)

REF FREQUENCY is the system reference frequency in Hz to the Phase

Detector.  VCO / N and >= 10 * BW for 20dB suppression.

Optional DC GAIN KA is any DC Gain (Voltage Ratio) between the Detector

and the Intergrator or between the Integrator and the VCO.  Use care

not to saturate following stages.  Use non-inverting amplifiers.

Any Phase Reversals generated by the amplifiers will unstabilize the

Loop.

FREQ DIVISION N is the Frequency Division of VCO to Reference Frequency.

VCO TUNING SENSITIVITY KV is the VCO tuning rate in HZ/Volt.

This value is multiplied by 2 * PI in the program to provide the proper

units of RAD/Sec/Volt.

PHASE DETECTOR KD  Phase Detector Sensitivity in VOLTS/RAD.

Linear, Digital Phase/Freq Detectors  KD ~= .10 to .80 V/RAD

Table 1       REF2 Pg 55 , REF3 Pg 11

Detector #         KL

1           1

2          PI/2

3          PI

4          PI/2

5          PI

6          2*PI

REF SUPPRESSION in dBV  Reference Frequency attenuation by the Loop

Filter response.

NOISE (N,BW,FR)  Design aid for showing the relative amount in dB of

added noise due to the N Divisor modified by Loop BW to FR relationship.

Page 6

NOISE VCO  Noise Reduction (dBV) at .1 of the 3dB BW.  This is also

known as the VCO Error Tracking or Low Frequency Noise reduction due

to the VCO Filtering.

PHASE MARGIN  = ATN(WO*T2)-90                 (Active)

ATN(WO*T2)-ATN(WO*(T1+T2))    (Passive)

GAIN MARGIN LO & HI = Gain Margins > 20dB are desired.

GM LO   = 20*LOG(KO * T1) dB

GM HI   = 20*LOG(K * T2)  dB

where K = KA*KD*KV/N

KO = Open Loop OP-Amp Gain (KO = 1 for Passive)

BODE PLOT RESPONSES

LOOP FILTER RESPONSE (dBV) is displayed in White.  This displays the

Loop Filter Response as defined by the Loop Filter Components and

System Constraints.

F(jw) = KO*|1+T2*Wo| / |1+((KO*T1)+T2)*Wo|

OPEN LOOP GAIN  (dBV) is displayed in Red.  This displays the Open

Circuit	Loop Gain, specifically of importance is the 0dB crossover

point.

G(jw)H(jw) = K*F(jw)/jw

CLOSED LOOP GAIN (dBV) is displayed in Blue on the BODE Plot.

C.L.G. defined as G(jw) / (1 + G(jw)H(jw))

Noise VCO  Error Response (E/En) is displayed in Green.  This is the

Low Frequency Tracking (dBV) of the Loop due to the VCO filtering.

E/En defined as 1 / (1 + G(jw)H(jw))

Operation near F1 AND F2's geometric center obtains the highest phase

margin for the system but operation above and below is possible,

depending on design goals.

DYNAMIC BODE PLOTTING

This feature allows the designer to "move" around the plot.

Pressing the up arrow key will Zoom in and the down arrow key will

Zoom out by changing the magnitude scaling.  A range of +/- 15db to

+/- 150dB is provided.  Initial plotting defaults to +/- 60dB.

Pressing the left arrow key will shift the plot lower in Bandwidth while

pressing the right arrow key will shift the plot up in Bandwidth.  The

Plot always defaults to the Loop Bandwidth the first time.

Paging up will shift the plot up by 1/2 screen, while paging down will

shift the plot down 1/2 screen.

Scaling is automatic.  The Phase Margin Plot does not change in this

feature.

The "SHIFT + F1" key will cause a B&W Screen Dump to printer.

See DISPLAY.CFG configuration file for setting up your printer.

Page 7

MARKERS:

f1 = Lower Pole of Loop Filter  (Yellow)

f2 = Zero of Loop Filter        (Green)

fn = Natural Frequency          (Violet)

fx = Unity Gain Crossover Freq  (Red)

fr = Reference Frequency        (Cyan)

The Foreground and Scaling colors can be set in the PLL2.CFG file.  The

Plotting colors still remain the same, therefore care should be taken

in selection of the colors used.  The color range for each is 0 - 15.

PLL2.CFG

line 1 = foreground color.

line 2 = axis colors

line 3 = Closed Loop position.  0 = no N,  1 = with N

GENERAL DESIGN RULES:

T1 > T2 likewise  f1 < f2

f2 < fn < fx < f3

fr >> f3

A Warning sign will Flash after the BODE Plot is finished indicating

any problems and a general solution.

REFERENCES

REF1    PHASED-LOCKED LOOPS

(C) 1984 by McGraw-Hill, Inc.

Dr. Roland E. Best, Author

REF2    PHASELOCK TECHNIQUES

Second Edition (C) 1979 by John Wiley & Sons, Inc.

Floyd M. Gardner,  Author

REF3    DIGITAL PLL SYNTHESIZERS Theory and Design

(C) 1983 by Prentice-Hall, Inc.

Ulrich L. Rohde, Author

REF4    SOLID STATE RADIO ENGINEERING

(C) 1980 by John Wiley & Sons, Inc.

Herbert L. Krauss and Charles W. Bostian, Authors

REF5    MODERN COMMUNICATION CIRCUITS

(C) 1986 By McGraw-Hill, Inc.

Jack Smith, Author

REF6    MOTOROLA MECL DEVICE DATA  Q1/89 DL122 REV 4 Pgs 6-22 to 6-36

REF7    FREQUENCY SYNTHESIZERS Theory and Design  3rd ED.

(C) 1987 John Wiley & Sons, Inc.   Vadim Manassewitsch

REF8    PHASE-LOCKED LOOPS Theory, Design, and Applications  2nd ED.

(C) 1993 McGraw-Hill, Inc.   Roland E. Best, Author

SoftWare Innovations For Technology Enterprises (R) 1989